| Specifications |  |
| --- | --- |
| Type | • Rack mountable 19“ storage solution FSC FibreCAT SX80 • Communication to 2 Gbps Fibre Channel • Scalable, with maximum storage capacity of up to 28 TB • Delivery with all configuration and management tools included • Redundancy for controllers, switches, fans, power supply • Ensured delivery in every location, installation, configuration and training of the personnel on using the system |
|  | • UPS with capacity 3000 VA, FSC Primepower • Storage capacity 2TB • Rack mountable 19”, maximum 2 U • Module I/O Fiber Channel • Connection to 2 Gps Fibre Channel hosts • Redundant switch with minimum 8 Fibre Channel ports of 2 Gbps each • Compatibility of components included in the configuration • Support hot-plug for hard-disks • Support hot-plug for fans • Support hot-plug for controllers • Support hot-plug for sources |
|  | • 1 Gbps and 2 Gbps Fibre Channel support for infrastructure • Hard disk configurating software tools enabling visualization and configuration of controllers and storage system • Software solution for tracking controller and hard disk status and indicating problems before malfunction occurs • Compatibility with servers with the following series: HP ProLiant ML570, ML370, IBM x235, IBM x255 , IBM x230, IBM x220 • Compatible with Windows 2000 Server, Windows 2003 Server, Linux • Unrestricted number of servers connected to the disk matrix by way of licenses or other additional requests • Possibility to configure the maximum disk capacity without requiring purchasing or obtaining additional licenses |
| Controller RAID | • Optical indicators of operating status • Minimum 512 Mb cache/controller (1 Gb / array) • Mirror cache • Support for RAID 5 • Enables access of several hosts to storage space • Interface compatible with the hard disks provided in the configuration |
| Fibre Channel | • redundant switch • 8 ports of 2 Gbps each, of which one will be used to connect the I/O module and the rest for the servers |
| Fibre Channel Adaptors | • Will be installed on the existing servers of the following types: HP ProLiant ML570, ML370, IBM x235, IBM x255, IBM x230, IBM x220 • 2 Gbps communication • Two adaptors for every location will be delivered and installed, and 7 adaptors on 7 servers will be delivered and installed at the head office of GPO and HCCJ (171 locations x 2 adaptors/each location = 342 adaptors; 2 locations x 7 adaptors/each location = 14 adaptors). Total no. of adaptors: 356. |
| Hard-disks | • The number of HDD required to ensure 2TB capacity of use will be installed • Rotation speed of 15,000 r/min • Interface U320 SCSI or SAS or Fibre Channel |